|  | Description | Qty | Unit | Rate ( RM ) | Amount ( RM ) |
| --- | --- | --- | --- | --- | --- |
| 1 | Concealed Ceiling with 12.5mm Gyproc Moisture Resistant Boards To supply and install Concealed Ceiling comprising of single layer Gyproc 12.5mm Moisture Resistant board size: 1220mm x 2440mm staggered screw-fixed not exceeding 200mm centres onto SteelMax Lightweight Secondary Channel “SLC2900” (BMT=0.32mm, Galvanised Steel) spaced at 609mm centres that are locked onto SteelMax Lightweight Main runner “SLM2900” (BMT=0.6mm, Galvanised Steel) spaced at 1200mm centres. The Main channels”SLM2900” are held to ceiling with Suspension Threaded Rod ¼” in diameter(length not exceeding 2m) which are fixed onto the soffit using ¼” drop in anchor at 1000mm centres; including SteelMax Perimeter “SLP2440” suspension to finished with SteelMax Secondary Channel “SLC2900” All boards joints secure with Gyproc Paper Tape and finished with Protop Joint Compound for a seamless finish. All fixing accessories, additional nogging for services installation as per manufacturer details | Rate | M² |  |  |
|  | GRAND TOTAL |  |  |  |  |